# EGR 431

# CONTROL SYSTEMS

Prepared for

Project #2 – DC Speed Controller

Prepared By

Andrew Willis 774959

Instructor: Dr. Jonsup Park
Project Performed on:
April 7, 2026
Report Submitted on:
April 9, 2026

Contents

Objective	3

Introduction and Background Theory	3

Project Requirements and Procedure	4

Requirements	4

Procedure	4

Theoretical / Simulated / Calculated Data & Discussion	5

Part A – Open-Loop Behavior	5

Part B – Closed-Loop Behavior	7

Case 1: ωmref=0,  Ka=5	8

Case 2: ωmref=140,  Ka=5	9

Effect of Increasing Controller Gain	9

Comparison and Interpretation	10

Conclusion	10

Appendix A – MATLAB CODE:	11

# Objective

The objective of this project was to analyze and compare the performance of an armature-controlled DC motor under both open-loop and closed-loop control configurations. The focus was to evaluate the effect of a disturbance torque on motor speed and to investigate how feedback control can reduce or eliminate this effect. MATLAB and Simulink were used to model the system, simulate its response, and analyze steady-state behavior and disturbance rejection characteristics.

# Introduction and Background Theory

Control systems are used to regulate dynamic systems and improve performance in the presence of disturbances. A disturbance is an unwanted input that negatively affects system output. One of the primary advantages of feedback control is its ability to reduce the impact of such disturbances and maintain desired system behavior.

In this project, an armature-controlled DC motor is analyzed. The system consists of both electrical and mechanical dynamics, which are described respectively:

Ea​(s)=(La​s+Ra​)Ia​(s)+Kb​ωm​(s)

(Jm​s+Bm​)ωm​(s)=Ki​Ia​(s)-TL​(s)

By eliminating the armature current, the motor speed can be expressed as:

ωm(s)=KiEa(s)-(Las+Ra)TL(s)(Jms+Bm)(Las+Ra)+KiKb

This system can be separated into two transfer functions:

Voltage-to-speed:

Gv(s)=Ki(Jms+Bm)(Las+Ra)+KiKb

Disturbance-to-speed:

Gd(s)=-Las+Ra(Jms+Bm)(Las+Ra)+KiKb

In the open-loop configuration, the system operates without feedback, and disturbances directly affect the output. In the closed-loop configuration, feedback is introduced using a sensor and amplifier gain, allowing the system to correct deviations from the desired speed.

The closed-loop disturbance transfer function becomes:

ωm(s)TL(s)=-Las+Ra(Jms+Bm)(Las+Ra)+KiKb+KaKsKi

The additional term in the denominator increases system gain and improves disturbance rejection.

# Project Requirements and Procedure

### Requirements

- Analyze the effect of a disturbance torque on motor speed

- Apply a step disturbance of 0.1 Nm at t=2seconds

- Simulate system response for:

  - Open-loop: Ea=10, Ea=20

  - Closed-loop: ωmref=0, ωmref=140

- Evaluate system performance for different controller gains:

  - Ka=5, Ka=40, Ka=100

- Determine steady-state error due to disturbance

- Compare open-loop and closed-loop performance

### Procedure

- The electrical and mechanical equations of the DC motor were derived.

- The armature current was eliminated to obtain a transfer function relating motor speed to both input voltage and disturbance torque.

- The system was separated into voltage-to-speed and disturbance-to-speed transfer functions.

- A Simulink model was developed for the open-loop system using these transfer functions.

- A step disturbance torque of 0.1 Nm was applied at t=2seconds.

- Simulations were performed for Ea=10and Ea=20, and steady-state values were recorded.

- The model was extended to include feedback, forming a closed-loop system using a sensor gain Ksand controller gain Ka.

- Simulations were performed for multiple reference speeds and controller gains.

- MATLAB was used to extract simulation data, calculate steady-state values, and compare system performance.

- Results from open-loop and closed-loop configurations were analyzed and interpreted.

# Theoretical / Simulated / Calculated Data & Discussion

### Part A – Open-Loop Behavior

Figure 1 - Open-Loop Simulink System

The open-loop system was simulated to evaluate the effect of a disturbance torque on motor speed. A step disturbance of 0.1 Nm was applied at t=2seconds.

For Ea=10, the motor reached an initial steady-state speed of approximately 125 rad/s. After the disturbance was applied, the speed decreased to approximately 110 rad/s, resulting in a steady-state change of about:

Δω≈-15 rad/s

For Ea=20, the motor reached an initial steady-state speed of approximately 250 rad/s. After the disturbance, the speed decreased to approximately 235 rad/s, again resulting in a similar change of:

Δω≈-15 rad/s

Figure 2 - Open-Loop Simulation

These results show that while the steady-state speed scales with input voltage, the disturbance effect remains approximately constant. This indicates that the disturbance response is independent of input magnitude in an open-loop system.

Additionally, once the disturbance is applied, the system settles to a new steady-state value without recovering. This demonstrates that open-loop systems lack the ability to compensate for disturbances, resulting in a permanent steady-state error.

## Part B – Closed-Loop Behavior

Figure 3 - Closed-Loop Simulink System

In Part B, the system was modified to include feedback, forming a closed-loop speed control system. The purpose of this configuration is to reduce the effect of disturbance torque and improve the system’s ability to maintain a desired reference speed.

The closed-loop system consists of a sensor with gain Ks=1, which measures the motor speed, and an amplifier with gain Ka, which acts as the controller. The measured speed is fed back and compared to the reference input ωmref, producing an error signal. This error is amplified and used to drive the motor input voltage.

The resulting closed-loop disturbance transfer function is:

ωm(s)TL(s)=-Las+Ra(Jms+Bm)(Las+Ra)+KiKb+KaKsKi

Compared to the open-loop case, the denominator now includes the additional term KaKsKi. This increases the overall system gain and effectively reduces the impact of the disturbance on the output.

### Case 1: ωmref=0,  Ka=5

Figure 4 - Closed Loop Simulation Case 1

In this case, the reference speed is set to zero. Ideally, the motor should remain at rest. When the disturbance torque is applied at t=2seconds, a small deviation in speed is observed due to the external load.

However, unlike the open-loop system, the closed-loop system detects this deviation through feedback and generates a corrective input. As a result, the motor speed quickly returns to zero after a short transient response.

This demonstrates that the closed-loop system is capable of actively rejecting disturbances and maintaining the desired operating condition even when external disturbances are present.

### Case 2: ωmref=140,  Ka=5

Figure 5 - Closed Loop Simulation Case 2

For a reference speed of 140 rad/s, the system reaches a steady-state value close to the desired speed. When the disturbance torque is applied at t=2seconds, the speed experiences a small drop.

Unlike the open-loop case, where the speed remains permanently reduced, the closed-loop system responds by increasing the input voltage to compensate for the disturbance. This results in the motor speed recovering and settling near the reference value.

Although a small steady-state error remains, it is significantly smaller than in the open-loop system. This confirms that feedback control improves system robustness and reduces the impact of disturbances.

### Effect of Increasing Controller Gain

To further evaluate system performance, the controller gain Kawas increased while maintaining a reference speed of 140 rad/s.

For Ka=40, the system exhibited a noticeably smaller deviation in speed when the disturbance was applied. The transient response was faster, and the steady-state error was reduced compared to the Ka=5case.

For Ka=100, the disturbance effect became minimal. The motor speed showed only a very small deviation from the reference and quickly returned to the desired value. The steady-state error was nearly eliminated.

These results indicate that increasing the controller gain improves disturbance rejection and enhances the system’s ability to maintain the reference speed. This occurs because higher gain increases the corrective action applied in response to error.

### Comparison and Interpretation

The closed-loop system demonstrates a significant improvement in performance compared to the open-loop configuration. In the open-loop system, disturbances directly affect the output, resulting in a permanent reduction in motor speed. In contrast, the closed-loop system continuously monitors the output and adjusts the input to counteract disturbances.

A direct comparison between open-loop and closed-loop performance highlights the advantages of feedback control. In the open-loop system, disturbances caused a permanent reduction in motor speed, regardless of input voltage. In contrast, the closed-loop system actively corrected for disturbances and maintained the desired speed.

The addition of feedback introduces a corrective mechanism that continuously adjusts the input to minimize error. As a result, the system becomes significantly more robust to disturbances.

Both MATLAB and Simulink results were consistent, validating the theoretical model and demonstrating the effectiveness of feedback control.

# Conclusion

This project demonstrated the behavior of a DC motor under both open-loop and closed-loop control conditions. In the open-loop configuration, disturbances caused a permanent reduction in speed, and the system was unable to recover due to the absence of feedback. The disturbance effect was shown to be independent of input voltage, highlighting a key limitation of open-loop control.

In contrast, the closed-loop system significantly improved performance by reducing the effect of disturbances and minimizing steady-state error. Increasing the controller gain further enhanced disturbance rejection, allowing the system to maintain the desired speed even in the presence of external loads.

Overall, this project highlights the importance of feedback in control systems and demonstrates how closed-loop control can effectively regulate system behavior and improve robustness in practical applications.

# Appendix A – MATLAB CODE:

%% Model Parameters function p = params() p = struct(); %% DC Motor Parameters p.Kb = 0.0772; p.Ki = 0.067; p.Ra = 0.7454; p.La = 4.8e-3; p.Jm = 6.87e-5; p.Bm = 0.0003; %% Controller / Sensor p.Ks = 1; p.Ka = 5; %% Disturbance p.TL_step = 0.1; p.Td = 2; %% Inputs (default) p.Ea = 10; p.wm_ref = 140; %% Simulation p.tstop = 6; p.den = [p.Jm * p.La, ... p.Jm * p.Ra + p.Bm * p.La, ... p.Bm * p.Ra + p.Ki * p.Kb]; p.num_v = [p.Ki]; p.num_d = [-p.La, -p.Ra]; %%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%% clc clear; close all; %% Load Parameters p = params(); %% Modify p.Ea = 20; %% Simulink model = 'sim_open'; simIn = Simulink.SimulationInput(model); simIn = simIn.setVariable('params', p); simIn = simIn.setModelParameter('StopTime', num2str(p.tstop)); simOut = sim(simIn); t = simOut.wm.Time; wm = simOut.wm.Data; info = stepinfo(t, wm); disp(info); plot(t, wm);